### CX3000 Family

LPGA, OneMask®, TwoMask® and HardArray™

0.35µ Low Power, Fast Time-to-Market Gate Array

###### The Next

###### Generation

Gate Array

###### Features

	Fastest gate array prototyping and turn-around times to production in the industry

- 1 Day for rapid-turn LPGA (laser pro- grammable gate array) prototypes

- 1 Week for OneMask pre-production quantities

- 1 Month for flexible TwoMask initial low-volume production ramp-up

- 8 Weeks for seamless transition to the low-cost HardArray™ family, for high-volume production

	High-density 0.35 architecture

- Up to 1.5M logic gates with up to 416K bits of embedded synchronous SRAM

- 10,000 HardArray gates/mm2, greater density than many standard cells

	Low power dissipation of 0.3W/MHz/gate

	Embedded high-speed memory

- Up to 416K bits of embedded synchronous SRAM

- 290 MHz worst case operation at 3V

- Configurable as FIFO, single or dual port SRAM or ROM

	More functions, features and I/O options

- Dual oxide process combines mixable 3.3 and 5 volt I/Os with 0.35 density

- Analog PLLs that minimize clock insertion delays

- Silicon layouts that minimize clock skew and signal cross-talk

- Differential I/Os include standby mode

- 3.3 volt PCI-compliant at 66MHz

- Support for GTL and USB

- Large cell, IP core and macrofunction libraries

- Support for Synopsys Design Compiler and Cadence Verilog XL

Description

The new CX3000 gate array family combines high-performance embedded memory blocks with an innovative architecture that can be laser programmed for delivery in a single day. Manufactured in an advanced dual-oxide 0.35 CMOS process, the CX3000 family features significant logic density improvement compared to conventional gate array architectures combined with low power dissipation (0.3W/MHz/ gate), while maintaining high speed operation. The CX3000 family provides up to 1.5M usable gates, with wide I/O capability, sophisticated IP cores and macro-functions, and a seamless transition to a very cost-effective solution for high-volume production requirements.

Power lines

Core

(550k modules,

1.5 Million Gates)

## I/Os

## (Up to 608)

Memory Blocks

(Up to 416k Bits)

## Analog PLLs

(Up to 4 Blocks)

CX3000 Device Layout

###### Cell Library

- Synopsys DesignWare™ support

- 30 low power flip-flops

- 37 built-in scan mux in flip-flops

- 74 flip-flops

- 20 latches

- Tri-state/drive cells

- Multiplexers, decoders

- 2308 I/O cells

- 11 power pads

- 7 ground pads

- IP microcontroller, DSP and macrofunction cores, and USB and GTL I/Os

###### Design Kits

CX3000 family libraries for popular EDA environments are available at no charge on the web site (www.chipexpress.com):

	Synopsys Design Compiler

	Cadence Verilog XL (sign-off simulator)

Design Literature

	User’s Design Guide

	Macro-cell Library Manual

	Synopsys Design Kit User’s Guide

	Verilog Design Kit User’s Guide

LPGA, OneMask and TwoMask ASICs

| Base | I/Os | Memory bits | Modules/ Logic Cells | Max Usable Gates |
| --- | --- | --- | --- | --- |
| CX3041 | 160 | 24K | 13K | 40K |
| CX3061 | 208 | 64K | 20K | 60K |
| CX3141 | 304 | 96K | 45K | 140K |
| CX3301 | 432 | 144K | 100K | 300K |
| CX3551 | 608 | 416K | 185K | 550K |

HardArray ASICs for High-Volume Production

| Base | I/Os | Memory bits | Modules | Available Gates |
| --- | --- | --- | --- | --- |
| CX3042 | 100 | 8K | 14K | 40K |
| CX3072 | 128 | 16K | 24K | 70K |
| CX3122 | 160 | 24K | 40K | 120K |
| CX3182 | 208 | 64K | 60K | 180K |
| CX3422 | 304 | 96K | 140K | 420K |
| CX3902 | 432 | 144K | 300K | 900K |
| CX31502 | 608 | 416K | 550K | 1.5M |

#### The QuICk® Laser System

Chip Express has developed a completely automated, highly accurate laser-based QuICk® prototyping system to provide fast-turn ASIC integrated circuits. This system combines patented technology and software. In order to personalize the ASIC devices, the laser system disconnects pre-defined metal links. Processing one die at a time, the system can remove 10 million links in 90 minutes. The processing of a 200K gate triple-metal device by the highly automated system takes less than two hours.

The laser uses automatic alignment, focus and scaling adaptation during operation. A real-time computer-driven image processing system drives the laser cutting process executing a pre-defined cut list of metal links. System lateral overall precision is 0.1 m during the on-the-fly cutting operation. The processing system incorporates an internal clean room compartment for die cutting.

For low volume production the disconnection is made at the wafer level. The system utilizes pro- prietary OneMask® and TwoMask® technology to personalize one wafer at a time to simultaneously etch multiple metal layers.

Chip Express Corporation

2323 Owen Street

Santa Clara, California 95054

Tel:  (408) 988-2445      Fax:  (408) 988-2449

email: moreinfo@chipx.com

http: //www.chipexpress.com

This information is Preliminary and subject to change without notice.

